# 1) Exterior, Hardware & User Interaction Enhancements

### - A molded holster integrated into the charger housing, surrounded by a uniform LED ring (like the design your team has in USA market)

### 2. Touch Buttons on the Body (for Polar series)

- Current control (+ / –)

- Start/stop/delay charging

Feedback:

- Many customers dislike depending fully on an app.

- Physical controls significantly reduce customer complaints as app access in not available in high rise buildings with basement parkings etc.

### 4. Thermal, electrical upgrades
Indian summers are hitting 45–50°C, PCB and varistors become extremely stressed and we have seen a higher failure rate of products during this season. Examples:

- Create airflow channels for PCB, using heat spreader, plates

- High-temperature-rated components (heatsinks,MOSFETs)

### - Improvement in Varistors & Capacitors for surge protection(Surge Protection Board)
One idea we had was If we can have Detachable module of input-side board with the varistors separate from main control board. If customer faces a power surge, only the input board with those varistors gets replaced rather than replacing the entire board

### 5. Adjustable CP (Control Pilot) Values From App

### Over last year, we have had a number of compatibility issues come from cars which in a lot of cases I think after diagnosis was repaired after software updates, for example in some chargers it was the Cp values which needed to be adjusted. So if we can have an option in the app to modify those values as per need it will help. Users can resolve issues themselves (e.g., older MG/ZS EVs, or problems we faced with Mahindra cars.
It can be a hidden “expert setting” that empowers technicians and reduces support cost.

### 6. Grounding Control: Multiple Levels + Disable Option

We have had cases where Charger’s earthing levels were not allowing charging, and in a couple of cases there were false ground faults which were later repaired via software update. In emergency cases where a customer is stranded with a portable/wall charger, it can be a problem. So apart from setting the earthing to low, medium, high, we can have an option to switch earthing off with a small disclaimer for it so that customer can decide for themselves.

### 7. Highly Accurate Energy Measurement (MID-like Precision)

Some customers have reported inaccurate consumption as well as power calculations in the app, resulting in refunds. If we can have a more accurate measurement of these values as it can be critical for locations like hotels who are collecting payments from customers based on those values.

### 8. Dynamic Load Management (DLM)
Enables multiple chargers or high-load appliances to coexist. Considering in India, average household load is around 5-7 kW, we have had cases where people were required to adjust charging speed, but a lot of times people want auto adjustment based on load available at home.

### 9. Multi-Charger DLM With Smart Algorithms
This is something more to do with public use projects, like corporate office where we have a number of chargers installed but the load available is limited. In that case power distribution becomes critical. So if we can have a way of distribution power among multiple chargers installed in a parking it helps maintain the balance.

### 10. Solar-Aware Charging Modes
A lot of chargers have started being used in parallel with solar. Considering a lot of on-grid solar benefits are being reduced, we have been shared queries to use chargers directly with solar as well. So if we can have option, where we can allow user to decide, whether to charge on solar only, or solar + grid mix, and grid only options. It helps them optimize charging costs. I think this feature is already available in a few products.

### 11. EV Insights: Battery %, Time & Range Prediction
If a user can enter car model, battery size, and starting battery % on the app, he can get an estimated battery % and better idea on the range. This can be a big differentiator as they get the data on public DC chargers, but not on AC chargers as of now.

### 12. Invite2Charge (Enhanced Edition)
We have the option to share the app access on multiple devices by sharing invites, we wanted to see if that access could be controlled  by the main account to limit time based use, or guest.

### 13. V2G (Vehicle-to-Grid) & V2H Readiness

Not for immediate use, but considering this is going to be a part of future iterations of the product. With ongoing grid modifications in some states in India, and customer at times wanting to power their home invertor also with it. we forsee that this will become an integral part of charging ecosystem down the line.